| Job Role Name QP Code (Level) | Assistant Bar Bender & Steel Fixer-CON/Q0202 | Batch Name/Id |  |
| --- | --- | --- | --- |
| Duration: | 9:00 AM to 5:00 PM | Max Marks: | 700 |
| Name of Candidate |  | Candidate Signature |  |
| Assessor Name |  | Aadhaar Number |  |
| Assessor Signature |  | Trainer Signature |  |
| TP Name |  | TC Name |  |
| TP/TC Stamp with Signature |  | Date |  |

| NOS | Theory |  | Practical & Viva |  |
| --- | --- | --- | --- | --- |
|  | Full Marks | Marks Obtained | Full Marks | Marks Obtained |
| CON/N0214 | 30 |  | 70 |  |
| CON/N0215 | 30 |  | 70 |  |
| CON/N0216 | 30 |  | 70 |  |
| CON/N0217 | 30 |  | 70 |  |
| CON/N0101 | 30 |  | 70 |  |
| CON/N8001 | 30 |  | 70 |  |
| CON/N9001 | 30 |  | 70 |  |
| Total | 210 |  | 490 |  |

Total Obtained Marks:..... /700

INSTRUCTIONS TO THE CANDIDATES

- No candidate will leave the examination Hall/Centre before the 17:00. If any candidate leaves center before 17:00 will be counted absent.

- Write your name, Aadhaar Number, father name, Signature clearly in the designated space.

- The enclosed paper contains 2 sections i.e. Theory & Practical Marks are mentioned against each question.

- Use of Calculators/ Mobile Phone is not permitted during assessment.

- Use only Black or Blue ball point pen. Don't do any rough work in paper.

- Do not mark multiple answers for one question, in case of multiple marking; you will not be given any marks for that particular question.

- To pass the qualification pack, every trainee should score a minimum of 50% of aggregate marks to successfully clear the assessment

CON/N0214: Read and understand reinforcement bar detail from hand sketches		(Marks:30)

Q1. What is standard length of TMT bar in feet?	 (6 Marks)

- 15 feet

- 30 feet

- 40 Feet

- 60 Feet

Q2.  Calculate the cut length of 8mm dia steel bar for making a stirrup size 230mm x 450mm? Nominal cover 40mm?	(6 Marks)

- 1264 mm

- 1294 mm

- 1200 mm

- 1300 mm

Q3. What is the Standard thickness of cover block for column?	(4 Marks)

- 25 MM

- 40 MM

- 50 MM

- 60 MM

Q4. What is the recommended cover for beams in marine structures?	(4 Marks)

- 25 mm

- 35 mm

- 45 mm

- 55 mm

Q5. Which type of binding wire is not used in reinforcement work??	 (4 Marks)

- Aluminum wire

- G.I wire

- M.S wire

- Both b & c

Q6. How will you calculate unit weight of steel?	 (6 Marks)

- D-2/162

- D-2/152

- D/162

- D-3/152

CON/N0215: Use and maintain materials/ tools and equipment relevant to reinforcement works 	(Marks:30)

Q7. Which of the following is not used for lifting and shifting of rebar?	 (6 Marks)

- Slings

- Safety Belt

- Lifting Belt

- Shackles

Q8. Identify the tool in given image?  	 (6 Marks)

- Rebar cutter

- Rebar Holder

- Wrench

- None of the above

Q9. Which of the following tools is used to bend and shape the rod as per required profile? 	 (8 Marks)

- Drilling Machine

- Bar bending machine

- Grinding Machine

- Bar cutting Machine

Q10. While shifting rebar manually, which group of basic PPEs must be used?	 (6 Marks)

- Safety shoe, safety helmet, safety belt, shoulders pack

- Safety shoe, safety helmet, safety belt, safety goggle

- Safety shoe, safety helmet, ear plug, safety goggle

- Safety shoe, safety helmet, hand glows, shoulder pack

| Assistant Bar Bender & Steel Fixer (CON/Q0202)– Level-2 |  |  |  |
| --- | --- | --- | --- |
| Practical Paper |  |  |  |
| CON/NO214: Read and understand reinforcement bar details from hand sketch. |  |  |  |
| Infrastructure & Tools: Measuring tape, hammer, spirit level, marking chalk/pencil, nails, line Dori, water tube, broom, trowel, rammer, straight edge, tile cutter, grinding and cutting tool, concrete vibrator, water tube, hand saw, jack planner, jack hammer, plumb bob concrete mixer, tile ,sand, cement, water |  |  |  |
| Sr No. | Practical skill assessment question | Marks | Duration (Min) |
| 1 | Consider fig.1 and find out the following details: a). Identify the RCC structure b). Identify the diameter of the main bars and stirrups c). Identify the shape of the cross section of the RCC structure c). Identify the shape of the cross section of the RCC structure | 20 | 60 Min |
| 2 | Find the spacing of the stirrups for the RCC structure shown in the fig no 1 | 8 |  |
| 3 | Identify and collect different size of cover block from the workshop's stores. And label them according to the type of RCC structure in which it is use (such as beam/column/slab). | 12 |  |
| 4 | Calculate the cutting length of following elements: | 20 |  |
| 5 | Determine the lap length for 16mm, 20mm and 25 mm bars. | 10 |  |
|  | Total Nos Marks | 70 |  |
| Note: Question not to be distributed |  |  |  |